# Built-in window louvre DucoGrille NightVent Manual

Manufactured by: DUCO Ventilation & Sun Control

This manually operated glass-replacing ventilation hatch (for glass thicknesses from 24 to 48 mm) is protected from outside by an aesthetic grille, with the option of a burglar-resistant version. Thanks to its flat inner panel, the DucoGrille NightVent boasts a sleek design.

## Features:

- Blade type: DucoGrille Solid 30Z with punching (P1) 21mm high x 2.5mm wide

- Louvre pitch: 37.5 mm

- Flange width: 30 mm

- Height: min. 350 mm, max. 2500 mm (depending on combination, see dimensions table in technical data sheet)

- Width: min. 350 mm, max. 2500 mm (depending on combination, see dimensions table in technical data sheet)

- Depth: 115 mm

- Weight: maximum 80 kg or if burglar-proof maximum 95 kg

- Material:

  - Inside and outside: aluminium Al Mg Si 0.5

  - Thermal break: PVC

  - Insulation: PIR

- Colours:

  - Interior panel: all RAL colours possible

  - Outer frame: all RAL colours possible

  - Lever: anodised F2

- Fitting: glass replacement

- Section thickness: minimum 1.2 mm

- Glass thickness: 24, 28, 32, 36, 40, 44 or 48 mm

- Visual free passage: 60 % (per metre of punching)

- Physical free area: 34 % (use the K-factor for calculation!)

- Passage: 369 L/s*m² @ 2 Pa

- Orientation: dropping open or hinged (left or right if width < 600 mm)

- Positions: 2 (open/closed)

- Stroke: see table in technical data sheet

- Operation: manual

- Operating conditions: -25° C to +60°C

## Accessories (included):

- Optional insect-repellent stainless steel mesh 2.3 x 2.3 mm

- Optional burglary protection according to class WK2

## Surface treatment:

- Anodisation: Qualanod-compliant, coating thickness 15-20µm, standard natural colour (colourless anodisation)

- Powder coating: Qualicoat Seaside type A compliant, minimum average coating thickness 60µm, standard RAL colours 70% gloss

Upon request: other finish coating thicknesses, anodising colours and paint gloss levels, textured paints and specific powder coating product codes

## Functional specifications:

### Flow rate:

  - K-factor inlet: 24.51

  - K-factor outlet: 24.51

  - Ce coefficient: 0.202

  - Cdcoefficient: 0.202

### Water resistance

  - v = 0.0m/s: class A

  - v = 0.5m/s: class B

  - v = 1.0m/s: class C

  - v = 1,5m/s: class D

  - v = 2.0m/s: class D

  - v = 2.5m/s: class D

  - v = 3.0m/s: class D

  - v = 3.5m/s: class D

### Attenuation value

  - Rw open position (C;Ctr) (in dB): 10 (0;-1)

  - Rw closed position (C;Ctr) (in dB): 33 (-1;-4)

### Insulation value

  - U = 1.5W/m²K

- Airtightness

  - Qn 50 overpressure = 3.6 m³/h/m²

  - Qn 50 underpressure = 2.4 m³/h/m²

  - Qn 100 underpressure = 5.4 m³/h/m²

  - Qn 100 underpressure = 3.8 m³/h/m²

## Complies with or tested in accordance with the following standards:

- Qualicoat Seaside type A (if painted finish)

- Qualanod (if anodised finish)

- EN 573 - EN AW-6063 T66 and EN AW-6060 T66: aluminium alloy & hardening

- EN 13030: water resistance and determination of Ce and Cd coefficients

- Class 3 - up to 600 Pa, EN 12207: air tightness

- Class 9a EN 12208, EN 1027: watertightness

- EN ISO 10140: acoustic measurements

- Class WK2: burglar resistance

- Class 2a EN 12210, EN 12211: resistance vs. wind load