# Transfer grille
DUCO Ventilation & Sun Control
DoorVent

## Description

The DoorVent is a transfer grille that can be installed discretely in internal doors. Unlike gaps under the door, the DoorVent thereby avoids draughts and attenuates intrusive noise.

## Dimensions

- Overall (width x height)			436 x 58 mm

- Recessed (width x height)			417 x 48 mm

- Door thickness				37‒47 mm

## Material and surface treatment

### Profiles

- Aluminium		EN AW-6063 T66

- Finish

  - Natural anodised (15-20 μm) according to Qualanod

  - Polyester powder coated (60-80 μm) according to Qualicoat Seaside type A (specific RAL codes or textured paint on request)

    - DUCO300RAL

### End caps

- Plastic

## Technical specifications

### Ventilation values

| Type | Air flow (in cm2) | at 1 Pa (in dm3/s) | at 2 Pa (in m3/h) | at 10 Pa (in m3/h) |
| --- | --- | --- | --- | --- |
| DoorVent | 70 | 4,7 | 25 | 54 |

### Sound reduction

| Type | Dn,e,W (C;Ctr) in dB | Dn,e,A (C;Ctr) in dB | Dn,e,Atr in dB |
| --- | --- | --- | --- |
| DoorVent | 32 (0;-1) | 32 | 31 |

According to NEN EN ISO 717

Please consult your regional dealer or the DUCO ‘Ventilation & Sun Control’ project department for commercial and technical support (mounting instructions, parts lists, calculations, …).