### Central mechanichal extract unit DucoBox Silent (Connect) UK

##### Description:

DucoBox Silent (Connect) UK, central mechanical extract ventilation unit, with humidity and/or CO2 sensor.

##### Operation:

DucoBox Silent Connect UK:

- Variable fan speed to reduce energy consumption and sound levels.

- Outline of components and modification of parameters possible via Duco Installation App & Duco Network Tool.

- Connect user controls (wired or wireless) with or without integrated humidity or CO2 sensors.

- Fault indication on user control, via wired communication only, using DUCO protocol with other control components by means of 5 x 0.8 mm² single core hard wire (power supply 2 x 0.8 mm² + communication 3 x 0.8 mm² (A-B-GND), or

- Wireless communication via RF protocol with other control components within the DUCO network (no fault indication).

- Duco Installation app assures faultless installation and generates ventilation report in line with regulations.

DucoBox Silent UK:

- Variable fan speed to reduce energy consumption and sound levels.

- Outline of components and modification of parameters possible via Duco Network Tool.

- Wireless communication via RF protocol with other control components within the DUCO network.

##### Material:

- Plastic: Polypropylene

- Electric motor: Supply voltage (VAC): 230

- Absorbed power (W): max. 84.39

- Type: direct current

- Thermally protected

##### Design:

- Colour: Green with white cover

- Connectors: Diameter (mm): 125

- Max. amount of connections on extraction side: 1

- Max. amount of connections on supply side: 7

##### Technical specifications

- Flow (m³/h): Max. 400

- Head (Pa): 150

- Dimensions (L x W x H) (mm): 480 x 480 x 195

- Weight: 4.3 kg

- Sound power level (dB(A)) (at 350 m³/h with a head of 100 Pa): Supply: 57.5, Emission from casing: 46.5

Accessories:

- Standard with G Plug (3-pin plug) power cord (3 x 0.75 mm²/2 m).

- Silent Plus Kit (optional):

- 3 acoustic blocks in central extract unit.

- 3 acoustic caps on central extract unit.

##### Application:

The DucoBox Silent (Connect), a mechanical extract ventilation unit, is often used in the following type of projects:

- Social housing

- Student accommodation

- Flats

- All other residential applications where there is natural air supply and mechanical extraction.

##### Fitting:

Fitting method: 	In an enclosed installation location against a wall with a mass > 200 kg/m²

Thanks to the measurements of the DucoBox Silent, it could also fit within a false ceiling.

Connectors: 	Extraction side: fit non-flexible, fixed ducts leading to the outside, the route of which should be as straight and direct as possible; increase in diameter: 125 mm to 160 mm. Use at least 2 extraction openings on the central extract unit, one for ventilating the kitchen and one for ventilating other rooms.

Connection method: 	Contact your regional distributor or the project department of DUCO Ventilation & Sun Control (info@duco.eu) for commercial and technical assistance.