# Solar shading system – DUCO Ventilation & Sun Control
DucoSun 100D Vertical

## Description

DucoSun 100D is an architectural component system with fixed "D" louvres. Thanks to the patented DUCO “Slide-Click” system, the louvre blades are secured quickly and easily in a fixed angle to the support structure.

## Version

- Shape of blade	100D

- Blade height		76 mm

- Blade depth		76 mm

- Blade angle		45°

- Pitch			127,5 mm

- Blade length		Maximum 6000 mm

- Installation depth

| Mullion | Installation depth |
| --- | --- |
| 50/12 | 94 mm |
| 21/50 Multi | 132 mm |
| 30/50 | 132 mm |
| 50/50 | 132 mm |
| 42/62 | 145 mm |
| 42/72 | 154 mm |
| 50/125 | 207 mm |

- Option		Coverplate (aluminium)

## Material and surface treatment

### Blades and mullions

- Aluminium		EN AW-6063 T66 (EN 573-3)

Profile thickness: min. 1,3 mm

- Finish

  - Natural anodised (15-20 μm) according to Qualanod

  - Polyester powder coated (60-80 μm) according to Qualicoat Seaside type A (specific RAL codes or textured paint on request)

### Clips

- Plastic			Polyamide PA 6.6 – glass fibre reinforced